“ MV  MATTEO BR ”

SHIP BASIC

YARD                    		ZHEJIANG AOLI SHIPBUILDING CO LTD - CHINA

DATE OF KEEL LAID       	26 AUGUST 2008

DATE OF DELIVERY/BUILT  	26 JUNE 2010

TYPE OF VESSEL          		GENERAL CARGO

FLAG                    		ITALIAN

PORT OF REGISTRY        	NAPOLI

CLASSIFICATION          		RINA CLASS REGISTER

TRADING                 		UNRESTRICTED AREA

IMO NUMBER              	9556844

CLASS NR                		88414

H+M INSURANCE COMPANY   	GENERALI

H+M VALUE               		20 MIL USD

PANDI                   		STANDARD P&I CLUB

DIMENSION AND MAIN DATA

LENGTH OVERALL          	157,00

LENGTH BETWEEN PP       	149,80

BREADTH MOULDED         	24,80

DEPTH MOULDED           	13,70

HEIGHT ABOVE KEEL       	40,00

GT/NT                   		15545/8149

SUEZ GT/NT              		15699,78/14766,01

DESIGNED DRAFT          	9,80

DWT                     		25035

TPC SUMMER DWT          	35,00

DISTANCE FROM WATER LINE TO TOP OF HATCHCOMING ABT 6.00 LOADED

DISTANCE FROM WATER LINE TO TOP OF HATCHCOMING ABT 10.60 BALLAST

DISTANCE FROM BOW TO BEGINNING FORE BULKHEAD HOLD NR 1 :  16,00

DISTANCE FROM STERN TO END OF AFT BULKHEAD   HOLD NR 4 :  31,20

HOLD/HATCHES

NUMBER OF HO/HA        	 4/4

DIMENSION

HATCH NR 1              		17,60 X 14,00           M

HATCH NR 2              		19,20 X 17,20           M

HATCH NR 3              		19,20 X 17,20           M

HATCH NR 4              		19,20 X 17,20           M

HOLD NR 1               		26 X 22 X 14            M

HOLD NR 2               		28 X 22 X 14            M

HOLD NR 3               		28 X 22 X 14            M

HOLD NR 4               		27 X 22 X 14            M

HEIGHT OF H/COAMING     	2,00

TYPE OF HATCHES         	FOLDING

HOLD/TANK CAPACITY ABT

CARGO HOLD ABT 		(GRAIN / BALE):

NR 1                    		245599 / 240687 CBFT

NR 2                    		286908 / 281170 CBFT

NR 3                    		286908 / 281170 CBFT

NR 4                    		272286 / 266841 CBFT

TOTAL                   		1091701 CBFT GRAIN

1069868 CBFT BALE

BALLAST                 		7256,40  CBM

F.WATER                		519.50   CBM

FUEL OIL                		891        CBM

DIESEL OIL              		215        CBM

LUB OIL                 		77,80    CBM

OTHER TANKS             		104,10   CBM

CARGO GEAR

TYPE                    		3 CRANES

SWL                     		30,5 MT EACH

SITUATED                		CENTRAL

RADIUS MIN/MAX          	4,5/24 M

DECK STREINGHT

TANK TOP                		18   MT / SQM

HATCH – COVERS          	1,75 MT / SQM

SPEED /CONSUMPTION

SPEED                   		ABT 11.5 KN / ABT 15 TS ULSFO

ISO 8217/2024  + 0.8TS MGO MAX 0.1 SULF DMA SPEC

BSS MAX BEAUFORT 4 NO SWELL AND NO ADVERSE CURRENTS

IN PORT W/OUT GEAR      	0.8   MT GASOIL

IN PORT WITH GEAR       	2.4   MT GASOIL

VSL BURN GASOIL ON RESTRICTED AREA / RIVER / CANAL AND ON PORT MANUVRING

VSL MUST USE MGO LOW SULFHUR FOR EUROPEAN PORTS AND SECA AREA

VENTILATION

HOLD                    		MECHANICAL , 6 AIR CHANGES PER HOUR

ADDITIONAL INFO

APP A/B/C FITTED

CO2 FITTED IN CARGO HOLDS

LOADING COMPUTER

ISM/ISPS CERTIFIED

GRAIN FITTED

ALL DETAILS TO BE ABOUT AND GIVEN WITHOUT GUARANTEE